30.12.2  Home Science Paper 2 (441/2)

| 1. | Presentation |  |  |
| --- | --- | --- | --- |
|  | (a) | Work well pressed and folded. | (1½ marks) |
|  | (b) | Label firmly fixed without concealing details on a single fabric. | (2½ marks) |
|  | (c) | Pins/needles, unnecessary tacking threads and loose threads removed. | (1½ mark)s |
|  | (d) | Made up for the left side. | (1 mark) |
| 2. | Cutting Out |  |  |
|  | (a) | All five pieces cut out. | (2½ marks) |
|  | (b) | Smooth cutting of the blouse C.B. and cut on straight grain to within 2 mm. | (3 marks) |
|  | (c) | Smooth cutting of collar C.B. and cut on straight grain. | (3 marks) |
|  | (d) | Smooth cutting of the armhole. | (4 marks) |
| 3. | Shoulder Seam (Open Seam) |  |  |
|  | (a) | Seam joined with straight stitchery. | (1 mark) |
|  | (b) | Well neatened raw edges. | (2 marks) |
|  | (c) | Evenness of seam allowance. | (1 mark) |
|  | (d) | Correct size of seam allowance (0.8 cm – 1.2 cm). | (1 mark) |
|  | (e) | Seam pressed open and flat at the neckline. | (1 mark) |
| 4. | Side Seam (French Seam) |  |  |
|  | (a) | Seam well trimmed that is, no threads on the R.S. | (1 mark) |
|  | (b) | Straight stitchery. | (2 marks) |
|  | (c) | Seam well knifed edged. | (1 mark) |
|  | (d) | Evenness of seam. | (2 marks) |
|  | (e) | Correct size (6mm) to within 2mm (4-8mm). | (2 marks) |
|  | (f) | Seam pressed towards the back at the hem. | (1 mark) |
|  | (g) | Flatness of seam at hem on R.S. and on W.S. | (1 mark) |
| 5. | Collar |  |  |
|  | (a) | Smooth outer edge of collar. | (2 marks) |
|  | (b) | Collar seam trimmed and snipped. | (2 marks) |
|  | (c) | Collar well knife edged with a sharp point. | (2 marks) |
|  | (d) | Correct shape and size (superimpose the collar pattern) to within 5mm. | (2 marks) |
|  | (e) | Flatness of attached collar R.S. and W.S. | (2 marks) |
|  | (f) | Collar attached by sandwiching it between blouse front and facing. | (1 mark) |
|  | (g) | Attaching seam made with straight stitchery. | (1 mark) |
|  | (h) | Seam trimmed and snipped. | (2 marks) |
|  | (i) | Positioning of collar C.F. of blouse flash with C.F. of collar and C.B. of blouse flash with C.B. of collar to within 3mm. | (2 marks) |
|  | (j) | Quality of hemming stitches. | (2 marks) |
|  | (k) | Quality of neatening the free edge of facing. | (2 marks) |
|  | (l) | Quality of diagonal stitches. | (2 marks) |
|  | (m) | Facing firmly held at the shoulder seam. | (2 marks) |
| 6. | Patch Pocket |  |  |
|  | (a) | Hem correctly finished, that is, raw edges well tucked under. | (1 mark) |
|  | (b) | Straight stitchery of pocket hem. | (1 mark) |
|  | (c) | Attached with straight stitchery and close to the edge. | (3 marks) |
|  | (d) | Raw edges of seam well tucked under. | (1½ marks) |
|  | (e) | Seam pivoted at the corners. | (1 mark) |
|  | (f) | Pocket mouth appropriately reinforced. | (2 marks) |
|  | (g) | Seam allowance evenly trimmed and snipped at the corners. | (4 marks) |
|  | (h) | Flatness of attached pocket on the R.S. and W.S. | (2 marks) |
|  | (i) | Correct shape and size of attached pocket (superimpose pattern piece to within 5mm). | (2 marks) |
| 7. | Blouse Hem (Machine Stitched) |  |  |
|  | (a) | Straight machine stitchery. | (2 marks) |
|  | (b) | Even tacks appropriately made:- - Close to fold. - Straight. - Raw edges well tucked under. | (3 marks) |
|  | (c) | Evenness of hem. | (2 marks) |
|  | (d) | Size of hem, 1cm to within 2cm. | (2 marks) |
|  | (e) | Flatness of hem. | (2 marks) |
|  | (f) | Hem appropriately finished at the C.F. with a pointed corner trimmed. | (2½ marks) |