30.16	METALWORK (445)

    - Metalwork Paper 1 (445/1)

1.	(a)

    - To protect eyes.

    - To protect feet.

    - To protect hands.

    - To protect clothes	.						(4 × ½ marks)

(b)	Public: are manufacturing industries which are either partially or wholly owned by the government.  Private: are industries owned by individuals (sole proprietor) or a group of individuals.								(2 marks)

2.	(a)

- Length.

- Cut.

- Shape. (cross section)

- Grade.								(3 ×  ½ marks)

(b)	(i)	Cross cut

- Cutting square holes.

- Making slots.

- Making key ways.

- Making channels.

(ii)	Half round

- Filing flat surfaces.

- Filing concave surfaces.						(Any 4 × ½ marks)

3.	(a)

    - Drawing circles and radii where ordinary compasses are too small.

    - Bisecting long lines.

    - Transfer of measurements.

    - Marking parallel lines to an edge.

    - Stepping equal distances.						(Any 4 × ½ marks)

(b)

(1 ½ marks)

4.	Physical :-Characteristics of metal which do not influence the strength of metal for example:- Colour, density, conductivity.  Mechanical:- Characteristic of metal which influence its strength for example:- Hardness, turfness, brittleness, malleability, elasticity etc.	(4 marks)

5.	(a)

(i)

- Backfire:-a condition whereby the flame momentarily recedes back into the tip before being expelled with a loud sound.

- Causes:-

- Weak acetylene frame.

- Leaks.

- Overheated torch.

- Dirty tip.

- Tip coming into contact with molten pool.				(Any 2× ½=1 marks)

(ii)

- Flashback:- the torch stays lit instead of expelling the frame.  It tends to travel in the torch body.

- Causes:-

- Improper pressures.

- Faulty mixing chamber.

- Overheating of torch.

- Faulty tip.

- Kinked hoses.								(Any 2× ½=1 marks)

6.

- Taking external measurements

(1 ½ marks)

- Taking internal measurements

(1 ½ marks)

- Taking depth measurements

(1 ½ marks)

- Taking stepped measurements

(1 ½ marks)

7.

(3 marks)

8.	(a)

- Both use	-	different working temperature.

-	different filler rod.

- Soldering	-	different flux.

- Brazed joint is stronger than soldered.					(2 marks)

(b)

- By bending and flattening.

- By wire edging (bend over a wire and retain it).

- By heading (bend over wire and remove it).				(3 marks)

9.	(a)

    - Pop rivet faster to install than snap rivet.

    - Pop rivet is done from only one side.					(2 marks)

(b)

- Protects surfaces from damages.

- Provides additional beauty.

- Protection from corrosion/rusting.					(1 ½ marks)

10.

(5 marks)

11.

(15 marks)

12.	(a)

(5 marks)

(b)	(i)

  - Mark out the development as shown in the diagram.

  - Cut and file to shape and size.

  - Using a mallet and folding bars, bend the two sides.

  - Bend the back over the sides

  - Bend the flaps.

  - Deburr the work piece.

(ii)

- Cut the required length of the flat bar.

- From a loop (eye) on one end of the handle.

- Bend the second end to correct shape and angle.

- Deburr the handle.

(iii)

- Clean the surfaces to be braced.

- Align the body and handle and lamp together.

- Open the cylinders and set the gas to correct pressure.

- Light and set the torch to correct flame.

- Heat the brazing rod and dip it into flux.

- Heat the joint to melting point of the rod.

- Braze the joint.

- Put off the flame.

- Clean the joint to remove excess flux.

- Shut off the cylinders and release the system pressure.	     		 (10 marks)

13.	(a)	(i)	Alloy of tin and lead: 	Should have:

- Low melting temperature.

- Low surface tension.

- High capillarity resistance to corrosion.

(ii)	Cast iron: Should be:

- Self lubricating.

- Hard surface.

- Easy to make.

(iii)	High speed steel: Should be:

- Resistance to rust and wear.

- Retain hardness even at high temperature.

- Hard.								(4½ marks)

(b)

(3×1½ marks)

(c)

- Bluing:- A method of finishing metal articles using heat to achieve a corrosion resistant surface.  Done by heating until colour changes to blue then dip the work into light oil and allow to cool.						(2 marks)

- Lacquering:- A process of metal finishing using lacquer for preservation and beauty.  The types of lacquer used include hot, gum cellulose and synthetic and is applied using a brush, dipping or spraying.					(2 marks)

- Planishing:- Is a process of finishing by making even decorative dents on sheets metal using a planishing hammer and stake.  The process includes annealing, picking buffing and cleaning.							(2 marks)

14.	(a)	(i)	Shank diameter

Ø = 1½ thickness = 3/2 × 4 = 6 mm

(ii)	Shank length = 2 × thickness + 1½Ø = (2 × 4) + (⅔ × 6) = 8 + 9 = 17

(iii)	Edge distance A = 1½D = 3/2 × 6 = 9

(iv)	pitch distance = 3D = 3 × 6 =18					(6½ marks )

(b)

    - Strength of the joint.

    - Thickness of the joint.

    - Appearance.

    - Where used.								(1½ marks)

(c)

    - Mark the holes and drill one hole on cover plate.

    - Drill a hole on one of the plates to be joined.

    - Debur the plates.

    - Cut the rivet to correct size.

    - Align the pieces and insert rivet.

    - Close the plates using rivet set.

    - Spread the tail of the rivet shank.

    - Form the head with ball pen.

    - Finish with rivet snap.

    - Drill the second hole and rivet.

    - Align the second plate.

    - Drill the holes for second plate.

    - Rivet the second plate.

    - Finish.								(14×½ marks)

15.	(a)

    - Cold forging procedures better finish than hot forging.

    - Cold forging work hardens and leaves the work stressed.

    - Cold forging requires ductile material and of small cross-section unlike hot forging.										(3 marks)

(b)	(i)

Eye:	D where D=20+3+3=35

35=×35=110

Straight part: 120-=102-20.5=99.5

Total length = 110+99.5=209.5 mm					(3 marks)

(ii)

- Mark the required for the eye.

- Bend the rod to 90°.

- Form the eye on anvil by start, further and closing.

- Hammer the eye on the anvil flatten.					(3 marks)

(iii)

- Hold the work piece in the vice.

- Chamfer the end to be threaded.

- Select the correct die M6.

- Fix the die in the die stock.

- Adjust the die to maximum opening.

- Fit the die square at the end of the bar.

- Apply cutting the thread.

- Continue cutting and reversing to beak the chips.

- Remove the die.

- Adjust the depth of the cut.

- Repeat thread cutting until the right depth is achieved.			(6 marks)