30.18.2  Power Mechanics Paper 2 (447/2)

Station 1

(10 marks)

Station 3

|  | Item | Material | Use |
| --- | --- | --- | --- |
| A | Exhaust Gasket. | Asbestos with copper or aluminium. | Seals between block and exhaust maniforld. |
| B | O ring. | Neopene rubber. | Prevents leakage between shafts and housing. |
| C | Brake fluid. | Vegetable oil. | Transmission of force in braking system. |
| D | Grease. | Mineral oil. | Lubrication of gears. |
| E | Oil strainer. | Steel. | Filter oil. |

(10 marks)

Station 6

| Part | Name | Engine System | Identified Fault | Possible Cause |
| --- | --- | --- | --- | --- |
| J | Commutator end. | Charging. | Worn carbon. | Normal wear and tear. |
| K | Mechanical fuel pump. | Fuel. | Inlett value wrongly installed. | Wrong assembly. |
| L | Shock absorber. | Suspension. | Worn out. | Excessive use. |
| M | Engine mounting. | Chasis. | Worn out or broken. | Excessive vibrations. |
| N | Tie rod end. | Steering. | Worn out. | Rough ride. |

(10 marks)

Station 8

- Material A: Filler rod used to fill joint when brazing or welding.

B: Flux used to clean metals and prevents oxidation or corrosion.

(2 marks)

Station 9

(a)	 (i)	Depth of keyway.

(ii)	Length of keyway.								(2 marks)

- (i)	Diameter of crankpin.

(ii)	Diameter of camshaft.							(2 marks)

- Crankshaft teeth.

Camshaft teeth.								(4 marks)

Ratio :