4.20	ELECTRICITY (448)

4.20.1  Electricity Paper 1 (448/1)

SECTION A

1.	(a)	Procedure of connecting an ammeter to take measurements in a circuit

-	Turn - off the power

-	Ammeter should be connected in series with the load current.

-	Observe polarity.

-	Select the range starting from the highest.

(b)	(i)	Nominal resistance

(4 #

1

2

= 2 marks)

Orange	Black	Brown

3	0	x	101	=  300 Ω

`  Nominal = 300 Ω	(1 mark)

(ii)	Maximum resistance

300 +  5% =  315 Ω	(2 marks)

2.	(a)	Circuit diagram

-	Shows connection of every component.

-	Shows values of components.

-	Shows the position of the components.

-	Shows functionality of the circuit.

(b)	Bills of materials

(any 2 # 1 = 2 marks)

-	Materials/parts.

-	Quantity.

-	Size.

-	Estimate costs.

(4 #

3.	(a)	(i)	Forward bias

reduces ^ 21 h the PN-junction (depletion layer) and hence the diode

conducts ^ 21 h.

1

2

= 2 marks)

(ii)	Reverse bias

increases ^ 21 h the PN-junction (depletion layer) hence the diode does

not conduct ^ 21 h.

534

(2 marks)

(b)	(i)	IF(max):  is the maximum forward current that the diode can pass without

burning out.	(1 mark)

(ii)	VF(typ):  is the forward voltage across the diode at the typical operating

current.	(1 mark)

4.	(a)	Ip =  V

R

=  100 Vrms

1  kX

= 0.1 A

^ 21 h

^ 21 h

^ 21 h

(b)	N1I1 = N2I2	(1)

I2 =	= 0.3 A

` 1200 # 0.1 = 400 # I2

120

400

V2 = I2 R2

= 0.3 x 8000

^ 21 h

^ 21 h

^ 21 h

^ 21 h

= 2,400

^ 21 h

(5 marks)

5.	(a)	(b)

Drawing	=	^ 21 h	Drawing	=	^ 21 h

Labelling	=	^ 21 h	Labelling	=	^ 21 h

Direction	=	^ 21 h	Direction	=	^ 21 h

=	1^ 21 h marks	=	1^ 21 h marks

6.	(a)	(i)	E = 5 + ^I # R1h

= 5 + ^2 # 10-3 # 2000h

= 5 + 4

=9V

535

^ 21 h

^ 21 h

^ 21 h

(ii)	R2 =  V2

I

^ 21 h

=    4V

2mA

^ 21 h

= 2 kX

^ 21 h

R3 =	=	1V   = 0.5 k

(iii)

V3

I	2mA

(b)	(i)	Energy consumed

Lights  5 # 60 # 4 = 1.2 kwh

Kettle  1 # 2 # 0.5 = 1.0 kwh

(1)

^ 21 h

^ 21 h

(4 marks)

Total energy = 2.2 kwh	(1)

(ii)	Cost of energy

= 2.2 # 80 = 1.76 sh	(1)

7.	(a)	Safety precautions to be observed

(3 marks)

-	Ensure that the equipment is properly earthed.

-	Do not use it in damp areas.

-	Always remove the plug from the socket when the equipment is not in use.

-	When using extensions, ensure the joints are firm and insulated using the

electricians insulation tape.

-	Hold it firmly.

-	Avoid loose clothing like ties.

(any 3 # 1 = 3 marks)

(b)	Communication service providers in Kenya

-	Telkom Kenya

-	Safaricom

-	Airtel

-	Yu

8.	(a)	Insulating materials used in electrical circuits

(4 #  21 = 2 marks)

or any other existing ones

-	PVC

-	Porcelain

-	Magnesium oxide

-	Paper

-	Rubber

-	Air

-	Formica

(4 #

1

2

= 2 marks)

536

(b)	Advantages of PVC

-	Easy of erection.

-	It is cheap.

-	It is resistant to corrosion.

-	It is light.

-	There is no risk to earth leaks.

9.	(a)	Inductance required

(any 3 # 1 = 3 marks)

4r2 f C

L =

1

2

^ 21 h

= 1.13 # 10-3 H

= 1.13 H

(b)	(i)	Apparent power

= IV

= 2.5 # 240

= 600 VA

(ii)	True power

^ 21 h

^ 21 h

^ 21 h

^ 21 h

1

(2 marks)

= apparent power # power factor

= 600 # 0.6

^ 21 h

^ 21 h

10.

= 360 w

^ 21 h

(3 marks)

Neatness	=

Proportionality	=

Faces	=	4 #

Holes	=	2 #

Projection	=	1

1

2

1

2

1

2

1

2

= 2

= 1

(5 marks)

537

SECTION B

11.

538

12.	(a)	Name of waveforms

A - sine wave

B - saw tooth

^ 21 h

^ 21 h

(b)	Number of cycles

A - 2 cycles	(1)

B - 3 41  cycles	(1)

(c)	(i)	Frequency of waveform A

=  1  where T = period

T

T = 50n # 4

^ 21 h

(1)

= 200 μs	(1)

^ 21 h

= 5 kHz	(1)

(ii)	Amplitude

A = Vpk = 200 mV # 3	(1)

= 600 mV	(1)

= 0.6 V

B = Vpk = 0.5 V # 2	(1)

= 1 Vpk	(1)

(iii)	RMS value of A

= 0.707 # Vpk	(1)

= 0.707 # 0.6

= 0.424 V	(1)

539

(13 marks)

13.	(a)

Correct Drawing	=	8

Labelling 6 items	=	3

(b)	(i)	Lighting circuit	=	5A

(ii)	Ring circuit	=	30 A

(iii)	Water heater	=	20 A

(iv)	Door bells	=	5 A

(v)	Cooker unit	=	45 A

540

(11 marks)

(any 4 #  21 = 2 marks)

(Total = 11 + 2 = 13 marks)

14.	(a)	(i)	Type of transistor

NPN	(1)

(ii)	Function of capacitor C

To block D.C	(1)

(iii)	Type of biasing

Fixed bias	(1)

(b)	(i)	VR1 = VCC - Vbe	(1)

= 6.0 - 0.6

= 5.4 V	(1)

(3 marks)

1

(ii)	IB =  VR1 =

5.4

100 # 103

(1)

= 5.4 # 105

= 54 μA

(iii)	IC = βIB	(1)

= 54 # 10-6 # 50	(1)

= 2.7 mA

(iv)	Voltage VCE

VR2 = IC # R2	(1)

= 2.7 mA # 1 # 103

= 2.7 V	(1)

VCE = VCC - VR2	(1)

= 6 - 2.7 V

= 3.3 V	(1)

541

15.	(a)	(i)	XL = 2rfL

= 2r # 50 # 0.05

= 15.70 Ω

(1)

^ 21 h

^ 21 h

XC =

1

2rfc

(1)

=

1

2r # 50 # 2 # 10-6

^ 21 h

= 1592 Ω

^ 21 h

Z =

=

R2 + ^XC - XLh2

10002 + ^1592 - 15.7h2

(1)

(1)

(ii)	Current =  V	(1)

= 1866 Ω	(1)

Z

(7 marks)

=   240

1866

(1)

(b)

= 0.12 A

^ 21 h      Amps  ^ 21 h      (1)

XL

(3 marks)

XC - XL

XC

ø

R

Z

V2

I

Axes	=	1

Labelling	=	4 #

1

2

= 2

(3 marks)

542