TERM 2 - 2023

PHYSICS – PRACTICAL (232/3)

FORM THREE (3)

Time – 212 Hours

Name …………………………………………….……… Admission Number …………….

Candidate’s Signature ………………….…...………..     Class ……………………………

INSTRUCTIONS:

- Answer all the questions in this paper

- You are supposed to spend the first 15 minutes of the 212 hours allowed for this paper reading the whole paper carefully before starting your work.

- Marks are given for clear record of the observations made, their suitability and accuracy and the use made of them.

- Candidates are advised to record observations as soon as they are made

- Mathematical table and electronic calculators may be used.

- The earth’s gravitational pull, g=10Nkg-1

For Examiner’s use only:

| QUESTION | TOTAL MARKS | CANDIDATE’S SCORE |
| --- | --- | --- |
| 1 | 20 |  |
| 2 | 20 |  |
| GRAND TOTAL | 40 |  |

This paper consists of 6 printed pages. Candidates should check the question paper to ensure that all the pages are printed as indicated and no questions are missing.

QUESTION 1

You are provided with the following apparatus:

- One dry cell

- A cell holder

- A volt-meter (0-3V)

- An ammeter (0-1A)

- A switch

- Amounted resistance wire labelled AB

- Micrometer screw-gauge

PROCEED AS FOLOWS:

- Set up the apparatus as shown in the circuit below, figure 1

Figure 1

- Using the micrometer screw-gauge provided, measure the diameter, D of the mounted wire.

D = .................................mm		D = ......................m		          (112 marks)

- Determine the cross-sectional area, A of the mounted wire given that:

A=πR2	where, R is the radius of the wire				             (2 marks)

- While the switch is open, record the voltmeter reading, V0

V0=	...........................							               (12 mark)

- Put on the switch. While the crocodile clip is at A (i.e. L = 100 cm) take the volt-meter reading (V) and the ammeter reading (I). Record V and I in the table, 1 below:

Table 1

| Length, L (cm) | 100 | 80 | 60 | 40 | 20 | 0 |
| --- | --- | --- | --- | --- | --- | --- |
| Voltage (V) |  |  |  |  |  |  |
| Current, I (A) |  |  |  |  |  |  |
| R = vI (Ω) |  |  |  |  |  |  |

(10 marks)

- Repeat the procedure in (c) above for the lengths shown and complete the table 1 above.

- Determine, Rav, the average value of resistance, R					  (2 marks)

- The relationship between resistance, R and length, l is given by the equation:

Rav=ρlA	, determine the value of the constant, ρ	when l=100cm	(3 marks)

- State the significance of the constant, ρ						(1 mark)

QUESTION TWO

You are provided with the following:

- a metre rule

- 3 optical pins

- 2 small wooden blocks

- a stop watch

- a stand, a boss and clamp

- a piece of sello-tape

Proceed as follows:

- Using the two wooden blocks, clamp two optical pins about 4 cm apart in the stand so that they project out of the blocks in a horizontal plane.

- Using a piece of sellotape, attach the third optical pin across the metre rule at a distance x = 10 cm from the 50 cm mark. Now suspend the metre rule on the two clamped pins so that it can swing freely in a vertical plan with the third pin as the axis. (See figure 2)

Figure 2

- Displace the lower end of the metre rule slightly and let it oscillate as shown in the figure 2. Measure and record in table 2 the time t (s) for 20 oscillations.

- Repeat the procedure in (b) and (c) for the values of x shown in table 2.

- For each value of x shown in the table, determine the period T(s), and complete the table. (The period T is the time for one complete oscillation).

(9 marks)

- On the grid provided, plot a graph of T2. X (y-axis) against X2			(5 marks)

- From the graph, determine:

- the slope S of the graph. 								(3 marks)

- The value of constant r given that: rS=39.5					(3 marks)

This is the last printed page