MOI UNIVERSITY

OFFICE OF THE DEPUTY VICE CHANCELLOR

(ACADEMICS, RESEARCH & EXTENSIONS )

UNIVERSITY EXAMINATIONS

2016/2017 ACADEMIC YEAR

SECOND YEAR SECOND SEMESTER EXAMINATION

FOR DIPLOMA

IN

INFORMATION TECHNOLOGY

COURSE CODE: 		DIT 028

COURSE TITLE: 		DIGITAL  ELECTRONICS

DATE :		 1ST SEPTEMBER, 2016 TIME: 2.00 A.M – 4.30 P.M

INSTRUCTIONS TO CANDIDATES

- SEE INSIDE.

THIS PAPER CONSISTS OF (3) PRINTED PAGES	               PLEASE TURN OVER

MOI UNIVERSITY

SCHOOL  OF INFORMATION SCIENCES

DEPARTMENT OF INFORMATION TECHNOLOGY

DIPLOMA IN INFORMATION TECHNOLOGY

END OF SEMESTER EXAMS – YEAR II SEMESTER II

2016/2017 ACADEMIC YEAR -  SEPTEMBER 2016 EXAMINATIONS

DIT 028: DIGITAL ELECTRONICS

Time: 2 hours 30 minutes

Instructions to candidates

- Section A is compulsory

- Answer any two questions in section B

SECTION A – COMPULSORY: ANSWER ALL QUESTIONS (30 MARKS)

QUESTION 1

- Explain any FOUR reasons for shifting from analog to digital transmission         (8 mks)

- Explain the following terms and state the SI units of each:                                     (6 mks)

- Resistance

- Inductance

- Capacitance

- With the aid of adiagram, describe the V-I characteristics of PN junction diode.   (6 mks)

- Derermine the total number of possible input combinations of a 3-input AND gate. Sketch a logic symbol and develop a truth table for this logic gate.                                                  (5 mks)

- An 8-bit DAC has an output voltage of the rage of 0-2.55v. define its resolution in two ways.

(3 mks)

- Convert 110012 to its equivalent decimal number.                                                     (2 mks)

SECTION B: ANSWER ANY TWO QUESTIONS (20 MARKS EACH)

QUESTION 2

- Show that X.Y.Z1 + X1.Y.Z1 +  Y.Z = Y                                                                    (4 mks)

- Evaluate the following:                                                                                               (6 mks)

- 2310 to Base Two

- 11011111002 to Base Eight

- A3F16 to Decimal

- Explain the following terms:                                                                                      (6 mks)

- Combination logic circuits

- Logic system

- Switching circuit

- Decoder

- Capacitor

QUESTION 3

- Explain three factors sffecting capacitance.                                                                  (6 mks)

- Briefly describe what is meant by the following terms.                                               (6 mks)

- Truth table

- Boolean expression.

- Universal gate

- Using a 2-input logic symbol and truth table, explain the meaning and logical operations of the following:                                                                                                                   (8 mks)

- NAND GATE

- NOR GATE

QUESTION 4

- Explain any FOUR applications of diodes.                                                               (8 mks)

- With the aid of a circuit diagrams, describe how transistor biasing is achieved.      (8 mks)

- State any FOUR advantages of transistors.                                                                (4 mks)

QUESTION 5

- State any TWO types of analog to digital converters (ADC).                                     (2 mks)

- State ohms law, and use it find the resistance of a coil which draws a current of 200uA from 120 V  supply.                                                                                                                       (4 mks)

- Find out the step size and analog input for a 4 bit R-2R ladder DAC when input is 1000 and 1111 assume vref =+5v                                                                                                          (6 mks)

- Explain any  FOUR characteristics of digital to analog converters.                             (8 mks)

=======END=======