TRIAL ONE EVALUATION TEST 2019

441/2     HOME SCIENCE       PAPER 2 MARKING SCHEME

- Presentation

- Work well pressed and well folded				1

- Label firmly stitched on single fabric				1

- Pins and tacking stitches/unnecessary threads removed	1

- Made up for the right half						1

Total	4

- Cutting out

- All 7 pieces cut out							7

- Smooth cutting of dress front skirt CF				2

- Straight grain of skirt CF to within 3 mm				1

- Smooth cutting of yoke CF						1

- Smooth grain of yoke CF to within 3 mm				1

- Smooth cutting of frills CF						1

- Straight grain of frills CF to within 3 mm				1

- Smooth cutting of dress back skirt CB				2

- Straight grain of skirt CB to within 3 mm				1

- Smooth cutting of back yoke CB 					1

- Straight grain of back yoke CB to within 3 mm			1

- Smooth cutting of frills CB						1

- Straight grain of frills CB						1

- Neck binding cut on bias (mark by impression)			1

Total	22

- Shoulder dart

- Straight stitchery tapering to appoint				1

- Thread ends well fastened at the point				1

- Correct length of the dart 4.6 cm to within 2 mm			1

- Correct width 7 mm to within 2 mm				1

- Dart flat on RS + WS							1

- Dart pressed toward neckline					1

Total	 6

- Joining the yokes to skirt

- Both yokes joined to skirt pieces using overlaid seam
(give zero if topstitched plain seam)					1

- Front yoke joined to skirt with straight stitchery close

to the  fold		                                                                                               1

- Front seam allowance even and unneatened				1

- Yoke and skirt flash at CF to within 3 mm					1

- Yoke and skirt flash at armhole to within 3 mm				1

- Back yoke and skirt joined with straight stitchery close

to the fold	                                                                                                             1

- Seam allowance unevenly trimmed						½

- Seam allowance well neatened						½

- Yoke and skirt flash at CB to within 3 mm					½

- Yoke and skirt flash at armhole to within 3 mm				½

Total	 8

- Side seam (French seam; if not, give zero)

- Well-made French seam with straight stitchery

(   stitchery)		                                                                                               1

- Well knife-edged								1

- Evenly trimmed (no threads/yarn on RS)					1

- Even width 0.5 cm to within 2 mm						1

- Flat at the frills RS x WS							1

- Seam pressed toward back							1

Total	 6

- Shoulder seam (Machine fell; if not give zero)

- Straight stitchery of the first line 						1

- Straight stitchery of the second stitching line close

to the fold		                                                                                               1

- Fell neatly tucked under (no yarn visible)					1

- Evenness of fell								1

- Correct width of fell 6 mm to within 2 mm					1

- Flatness of the seam RS + WS						1

- Seam worked on RS								1

- Fell pressed toward back							1

Total    	8

- Pocket

- Hem of the pocket even								1

- Hem fixed with straight stitchery							1

- Raw edges neatly tuck under all round 						1

- Pocket attached with good stitchery close to the fold

and reinforced at the mouth		                                                                                 1

- Pocket attached at the right position, that is, at about

5.5. cm from CF and at about 2 cm from the frills	                                                      1

- Flatness of pocket									1

- Correct size and shape to within 3 mm (use pattern piece

to superimpose)	                                                                                                            2

- Buttonhole cut on a straight grain							1

- Button hole made with correct stitch						1

- Quality of stitchery (tension, length and spacing

of the stitches)			                                                                                  3

- Size of the button hole								1

Total 14

- Frills

- Frills attached with straight stitchery						2

- Gathers evenly distributed								3

- Seam allowance left unneatened							1

- Frills and skirt flash at CB + CF							1

- Evenly made hem									2

- Hem fixed with good stitchery							3

- Flatness of the hem									2

Total	14

- Neck binding

- Evenness of the neck binding							2

- Raw edges neatly tucked under (no yarns visible)				2

- Good stitchery									2

- Edges flash with CB + CF								1

- Flatness of the binding								1

Total	8

Sum Total  90 / 2 =4 5 marks							90 /2